RACK & PINION
SELECTION REPORT

SHODA BS101C CNC Router — X/Y Axis Drive

| Document item | Description |
| --- | --- |
| Purpose | Step-by-step rack, pinion, gearbox and motor selection calculations. |
| Machine configuration | Moving gantry with two Y-axis rack-and-pinion drives, one per side. |
| Reference | User-provided Spec of BS101C.xlsx; specifications previously identified from Sheet2, GearBox and Rapid. |
| Status | Preliminary method; final tooth load capacity and service life are not certified. |

Important limitation: Actual moving gantry mass, acceleration profile, cutting-force envelope and supplier tooth-load ratings remain to be confirmed. This report defines the calculation workflow; it does not certify that the existing drive is adequate.

# 1. Executive summary

The identified workbook specifications describe a helical rack-and-pinion arrangement with module 2 (assumed normal module), 25 pinion teeth, listed pitch diameter 53.05 mm, and a 7:1 gearbox. The stated X/Y rapid traverse is 60 m/min. The machine uses two Y-axis drives, one at each side of the moving gantry.

| Parameter | Value identified | Design note |
| --- | --- | --- |
| Rack | Helical, C45E, induction hardened HRC 50–55, DIN 6 | Confirm supplier drawing and allowable load. |
| Pinion | Module 2, 25 teeth, S45C, HRC 50–55, DIN 6 | Confirm normal module, pressure angle and profile. |
| Pitch diameter | 53.05 mm | Consistent with normal module 2 and helix angle ~19.528°. |
| Gearbox | Liming SF75-1-110M8, 7:1 | Verify exact model and datasheet. |
| Gearbox efficiency | At least 97% listed | Use supplier-defined conditions. |
| Output torque | 142 N·m rated; 1.8× acceleration; 3× emergency-stop listed | Confirm definitions and allowable durations. |
| Motor | FANUC βiS 8/3000; 3,000 RPM max, 2,000 RPM rated listed | Verify exact motor code and torque-speed curve. |
| Rapid traverse | 60 m/min listed | Check torque at speed and acceleration. |
| Machine weight / spindle mass | 2,500 kg / 21 kg listed | Machine weight is not moving gantry mass. |

# 2. Moving mass — Step 1

Calculate each axis separately. For Y, include the gantry bridge, X-axis components carried by the gantry, Z assembly, spindle, carried motors/gearboxes, brackets and effective moving cable-chain mass. Exclude the stationary bed and fixed table.

mY = Σ mi

mX = Σ (components moving with X)     |     mZ = Σ (components moving with Z)

Use SolidWorks Evaluate → Mass Properties on a carefully selected moving assembly. Check material densities and avoid double-counting subassemblies. Do not use the listed 2,500 kg total machine weight as Y-axis moving mass.

# 3. Helical geometry — Step 2

If module 2 is the normal module mn:

mt = mn / cos β

dp = mt z = (mn z) / cos β

Using mn = 2 mm, z = 25 and β = 19°31′42″ ≈ 19.5283°:

dp = (2 × 25) / cos(19.5283°) ≈ 53.05 mm

rp = dp / 2 ≈ 26.525 mm = 0.026525 m

pn = π mn = 6.283 mm;     pt = π mt ≈ 6.666 mm

The listed diameter agrees if module 2 is the normal module. Confirm module convention, normal pressure angle, helix angle magnitude, opposite helix hands, tooth profile/profile shift and mounting orientation against the supplier drawing.

# 4. Rapid traverse — Step 3

V (m/min) = π dp (mm) Nm (RPM) / [1000 i]

With dp = 53.05 mm and i = 7:

At 3,000 RPM: V ≈ 71.43 m/min

At 2,000 RPM: V ≈ 47.62 m/min

Nm for 60 m/min = 1000 V i / (π dp) ≈ 2,520 RPM

60 m/min is kinematically possible at about 2,520 RPM. This does not prove the motor can deliver required torque at that speed or that the gearbox, bearings, lubrication and machine structure permit the operating point.

# 5. Acceleration and driving force — Step 4

a = (v − v0) / t;     v (m/s) = V (m/min) / 60

Fa = mY a

Ff = μ mY g, where g = 9.81 m/s² (simplified estimate only)

Freq = Fa + Ff + Fc + Fe

Fdesign = Ks Freq

Fc is cutting force opposing travel; Fe includes other resistance such as cable-chain drag, seal drag and preload-related resistance. For final sizing, use guide-manufacturer resistance data where available. A preliminary Ks = 1.5 was discussed, but it does not replace separate cutting, shock, emergency-stop and duty-cycle load cases. Include gravity components for inclined or vertical axes.

# 6. Load per Y-axis pinion — Step 5

Ideal equal split: Fpinion = Fdesign / 2

The 50/50 split is only a preliminary assumption. Confirm load sharing from gantry stiffness, rail/rack alignment, friction differences and drive synchronization. With two independent servos, size each side for its own peak and continuous load and verify electronic gantry synchronization/squaring. With one motor and a mechanical synchronizing shaft, calculate combined motor and transmission loads.

# 7. Pinion torque — Step 6

Tideal = Fpinion rp = Fpinion dp / 2

Tp = Fpinion dp / (2 ηr)

Use dp in metres to obtain N·m. For dp = 0.05305 m:

Tp = Fpinion × 0.05305 / (2 ηr)

If ηr = 0.95 is used only as a preliminary assumption:

Tp ≈ 0.02792 × Fpinion N·m

Use supplier efficiency where available and do not count the same losses twice.

# 8. Motor torque and power — Steps 7–8

Tm = Tp / (i ηg)

Using i = 7 and workbook-listed gearbox efficiency ηg = 0.97:

Tm = Tp / (7 × 0.97) ≈ 0.1473 Tp

P (kW) = T (N·m) × N (RPM) / 9550

Pm = Tm Nm / 9550

Prack (W) = Ftotal (N) × v (m/s)

These equations estimate torque and power at an operating point. Final servo sizing must include acceleration torque from rotating inertia, peak torque, continuous RMS torque over the duty cycle, thermal limits and the motor torque-speed curve. Apply efficiencies consistently.

# 9. Gearbox capacity — Step 9

| Workbook-listed rating | Arithmetic | Design use |
| --- | --- | --- |
| Rated output torque | 142 N·m | Nominal rating subject to datasheet conditions. |
| Acceleration torque | 1.8 × 142 = 255.6 N·m | Peak acceleration rating; confirm allowed duration/cycle. |
| Emergency-stop torque | 3 × 142 = 426 N·m | Emergency rating; confirm definition and permissible events. |

Do not use peak or emergency-stop ratings as continuous allowable torque. Verify input speed, radial/axial output loads, service factor, duty cycle, temperature, lubrication and life. The rack, pinion, shaft, key, coupling or bearings may have lower capacity than the gearbox.

# 10. Rack/pinion tooth strength — Step 10

Lewis-type screening only: σF ≈ Ft / (b m Y)

Ft is tangential tooth force (N); b is effective face width (mm); m is the module required by the selected method (mm); Y is a geometry-specific form factor. This simplified equation is not a final rating for this helical rack-and-pinion system.

Use the supplier's allowable tangential force/torque or an applicable gear-design method (such as ISO 6336 where applicable) with correct geometry and factors. Check:

Tooth-root bending strength and fatigue.

Flank contact stress, pitting and wear.

Material grade, hardness and effective hardening depth.

Face width, pressure angle, helix angle and profile shift.

Dynamic/load-distribution factors, shock and cutting loads.

Lubrication, contamination, alignment, rack joint steps and mounting stiffness.

# 11. Positioning and service life — Step 11

Gearbox, rack/pinion and coupling backlash contributions.

Required positioning accuracy/repeatability, feedback and servo tuning.

Gantry racking/skew risk and dual-drive synchronization.

Travel, cycles, acceleration frequency, load spectrum and required hours.

Lubrication interval and protection from wood dust/chips.

Rack joint alignment and mounting stiffness.

The listed gearbox backlash of no more than 1 arc-minute is only one contributor to machine positioning performance; it does not alone establish axis accuracy.

# 12. Inputs needed to complete numerical sizing

| Required input | Why it matters |
| --- | --- |
| Actual Y-axis moving mass from SolidWorks (kg) | Calculates acceleration force and inertia. |
| Acceleration or time to reach 60 m/min | Defines inertial force and peak torque. |
| Cutting-force design case in Y direction (N) | Adds machining load. |
| Drive arrangement | Two independent servos or one motor with mechanical synchronization. |
| Rack/pinion supplier load rating and drawing | Verifies geometry, allowable force/torque and tooth strength. |
| Exact motor torque-speed curve and motor code | Checks torque near 2,520 RPM and peak/continuous capability. |
| Duty cycle / required life | Supports thermal and fatigue verification. |
| Guide resistance and cable-chain drag | Replaces rough friction assumptions with actual loads. |

# 13. Recommended sign-off sequence

Confirm rack/pinion geometry and module convention from the supplier drawing.

Extract X, Y and Z moving masses from SolidWorks.

Define rapid, cutting, acceleration, deceleration and emergency-stop load cases.

Calculate total Y-axis force and realistic load sharing between the two pinions.

Calculate pinion torque, motor torque, speed, power and reflected inertia.

Verify motor and gearbox ratings against the complete motion profile.

Complete supplier-specific tooth bending/contact and service-life checks.

Review backlash, gantry synchronization, lubrication and installation requirements.

# 14. Formula reference

| Calculation | Formula |
| --- | --- |
| Moving mass | m = Σ mi |
| Acceleration | a = (v − v0) / t |
| Acceleration force | Fa = ma |
| Simplified horizontal friction | Ff = μmg |
| Total axis force | Freq = Fa + Ff + Fc + Fe |
| Design force | Fdesign = Ks Freq |
| Helical transverse module | mt = mn / cos β |
| Pitch diameter | dp = mt z = mn z / cos β |
| Linear speed | V = π dp Nm / (1000 i), dp in mm |
| Pinion torque incl. mesh loss | Tp = Fpinion dp / (2 ηr), dp in m |
| Motor torque through gearbox | Tm = Tp / (i ηg) |
| Power | P(kW) = T(N·m) × N(RPM) / 9550 |
| Rack translational power | P(W) = F(N) × v(m/s) |
| Tooth stress screening | σF ≈ Ft / (b m Y); not final helical gear rating |

# Engineering conclusion

The listed module 2, 25-tooth helical pinion and 53.05 mm pitch diameter are geometrically consistent if module 2 is the normal module. The 7:1 gearbox gives a theoretical 60 m/min speed at about 2,520 motor RPM. These checks alone do not prove the rack, pinion, gearbox or motor is adequately sized. Final approval requires the actual moving mass, dynamic load cases, exact motor/gearbox datasheets and supplier-specific rack-and-pinion load/life verification.

Source and assumptions: Prepared from specifications previously identified in the user-provided Spec of BS101C.xlsx workbook and the stated dual-drive moving-gantry configuration. Cross-check values against the original workbook, supplier drawings and current datasheets before design release.