# AUDIO METERING — QUICK REFERENCE

Cubase 12 mixing notes

# 1. Meter meanings

| Meter | Meaning | Remember |
| --- | --- | --- |
| Peak (dBFS) | Highest digital sample level | Loudest instant |
| True Peak (dBTP) | Estimated highest reconstructed waveform level | Hidden peak |
| RMS (dBFS) | Average signal energy over time | Average level |
| Crest Factor (dB) | Peak − RMS | Transient punch |
| Integrated LUFS | Perceived loudness of the full song | Overall loudness |

## Example to remember

Peak = −4 dBFS

True Peak = −3.2 dBTP

RMS = −16 dBFS

Crest Factor = −4 − (−16) = 12 dB

Integrated Loudness = −14 LUFS

Cubase mixing rule: Peak = avoid clipping; True Peak = check inter-sample peaks; RMS = help balance stems; Crest Factor = understand dynamics; LUFS = measure overall loudness.

# 2. Instrument meter quick reference

| Instrument | Primary meters | Main purpose |
| --- | --- | --- |
| Kick / snare | Peak, Crest Factor | Transients and punch |
| Tabla / mridangam | Peak, RMS | Strikes and average energy |
| Bass / sub-bass | RMS, Peak | Sustained energy and attacks |
| Lead vocal | RMS, Peak | Consistency and loud syllables |
| Backing vocals | RMS, Peak | Relative balance and spikes |
| Strings / pads | RMS | Sustained energy |
| Flute / melodic instruments | RMS, Peak | Average balance and attacks |
| Reverb / delay returns | Peak, RMS | Return level and buildup |
| Stereo Out / master | Peak, True Peak, LUFS, RMS | Headroom, reconstructed peaks and loudness |